Unique porous materials synthesized using template method

Hirotomo Nishihara*

Advanced Institute for Materials Research (WPI-AIMR), Tohoku University, 2-1-1, Katahira, Aoba-ku, Sendai, 980-8577, Japan

*hirotomo.nishihara.b1@tohoku.ac.jp

Abstract

The template method is a powerful technique for synthesizing a variety of porous materials with ordered structures and unique properties. In this talk, such unique porous materials are introduced, covering a rame from micropores to mesopores and macropores.

1. Ordered carbonaceous frameworks (OCFs) [1]

Through rational molecular design, it is possible to convert organic crystals into ordered carbonaceous frameowrks (OCFs). OCFs feature ordered micropores with single-atomic metal species embedded in the grahene-based framework. OCFs exhibit unique electrocatalysis and mechanical flexibility [2].

2. Graphene MesoSponge® (GMS) [3]

Graphene MesoSponge (GMS) is a unique mesoporous carbon composed primarily of single-graphene walls that are free from edge sites. Due to its developed mesopores, high electric conductivity, high oxidation resistance, and mechanical flexibility, GMS is expected for a variety of applications, including a new type of heat pumps [4], supercaiacitors [5], fule cells, and Li-O2 batteries [6, 7].

3. Carbon-coated anodic Al oxide [8]

Anodic Al oxides (AAOs) are know as oedered meso/macroporous membranes with cylindrical nanopores alighed perpendicular to the Al substrate. By removing the bottom insulator layer called barrier layer, followed by uniform carbon coating via chemical vapor deposition, it is possible to obtain conductive membrane electrode with uniform cylindorical nanopores, called carbon-coated AAOs (C/AAOs). C/AAOs are useful to elucidate the relation between carbon nanopores and electrochemical properties and performances [9].

4. Microhoneycomb monoliths [10]

When a solid/water mixture satisfies specific conditions, unidirectional freezing of the mixture induces an array of ice-crystal rods pallalel to the freezing direction. After lyophilization, a honeycomb-structured monolith with straight microchannels, known as microhoneycomb monoliths, can be obtained [11]. This approach can yeild highly peameable filters [12].

References

- H. Nishihara, et al., Nature communications, 8, 109 (2017).

- T. Yoshii, et al., Chemical Communications, 58, 3578-3590 (2022).

- H. Nishihara, et al., Advanced Functional Materials, 26, 6418-6427 (2016).

- K. Nomura, et al., Nature communications, 10, 2559 (2019).

- K. Nomura, et al., Energy & Environmental Science, 12, 1542-1549 (2019).

- W. Yu, et al., Advanced Energy Materials, 14, 2303055 (2024).

- W. Yu, et al., Advanced Science, 10, 2300268 (2023).

- H. Liu, et al., Advanced Functional Materials, 33, 2303730 (2023).

- H. Liu, et al., ACS Applied Materials & Interfaces, 15, 40397-40408 (2023).

- Z. Z. Pan, et al., ACS Nano, 10, 10689-10697 (2016).

- S. R. Mukai, et al., Chemical Communications, 874-875 (2004).

- M. Liu, et al., ACS Applied Materials & Interfaces, (2024).

Biography

Hirotomo Nishihara is a professor at Tohoku University. He received his PhD degree in 2005 from Kyoto University and subsequently served as an assistant professor at Tohoku University. In 2011, he was promoted to associate professor and in 2020, he was promoted to full professor. He also serves as an editorial board member for several international journals, including Scientific Reports, Energy Storage Materials, Carbon Research, and Carbon Reports. In 2022, he took his innovations into the business sphere by establishing a startup company, 3DC. As the Chief Scientific Officer, he is dedicated to the industrialization of a novel porous carbon material he developed. His research interests encompass carbon materials, nanoporous materials, adsorption, and energy storage, particularly in the field of batteries.